# 32-bit Processor Design

Zaman Ishtiyaq | 15CS01043 | Autumn Semester 2017

Contents

Overall Architecture

Instruction Format

Instruction Set

Components of Processor

Running an Example

# Overall Architecture

| - General purpose registers | 32 |
| --- | --- |
| - Clock cycles per instruction | 4 |
| - Instructions | 8 |
| - Memory | R.A.M (18-bit Address &32-bit Data) |
| - Special Registers | RA, RB, RC, RZ, RY |

# Instruction Format

OOO AAAAA BBBBB XXXXXXXXXXXXXXXXXXX

| OOO | OP-CODE |
| --- | --- |
| AAAAA | RA |
| BBBBB | RB |
| XXXXXXXXXXXXXXXXXXX | IMMEDIATE |

# Instruction Set

| OP-Code | Instruction | RTN |
| --- | --- | --- |
| 000 | LOAD RA, RB, IMMEDIATE | RB [ [RA]+IMMEDIATE ] |
| 001 | STORE* RA, RB, IMMEDIATE | [RB] [ [RA] +IMMEDIATE] |
| 010 | MOV RA, RB | RB [RA] |
| 011 | JUMP RA, RB, IMMEDIATE | PC IMMEDIATE |
| 100 | ADD RA, RB | RB [RA] + [RB] |
| 101 | SUBTRACT RA, RB | RB [RA] - [RB] |
| 110 | MULTIPLY RA, RB | RB [RA] * [RB] |
| 111 | DIVIDE RA, RB | RB [RA] / [RB] |

* STORE can only write to the RAM so the IMMEDTIATE value should be of the form 1XXXXXXXXXXXXXXXXXX as Chip-Select has to select RAM otherwise it will try to write on to the ROM and it will have no effect.

# Components of the Processor

Following are the components along with their figures:

- Processor Pipeline:

- Control Unit

- Fetch Unit:

- Fetch Unit increments the PC or jumps to a new address (Immediate Value) when JUMP command is given.

- Instruction Register and Instruction Decoder:

- ALU:

- Supports Addition, Subtraction, Multiplication and Division.

- ROM-RAM:

- We write our programs on ROM. ROM and RAM are selected based on Chip Select which changes on different operations.

ROM

- Register File (Internal Circuit):

- 9.     Reset Button     Resets Everything except ROM.

# Running an Example

Let us perform the following example:

Initially:

- Press the RED

- Change the Register Values in RF to:

R0: 1        R1: 3         R2: 7         R3: 6         R4: 2        R5: 1         R6: 2

- Let’s Load the ROM with the following values:

00000 :    46280000

00001 :     80080000

00002 :    a3200000

00003 :    c3280000

00004 :    e5080000

00005 :    0008000a

00006 :    202c0000

00007 :    6000000f

00008 :    00000000

.

.

.

0000b :   00000011

.

.

.

0000f :   80080000

00010 :   80080000

When Executed following happens:

Initially the RF looks like :

R0: 1        R1: 3         R2: 7         R3: 6         R4: 2        R5: 1         R6: 2

- 46280000   in binary is         01000110001010000000000000000000

i.e. MOV R6, R5

- Stage – 1: Fetch to IR

- Stage – 2: RA gets value from R6

RA  [R6]

- Stage – 3: RA gets value from R6

RA’s value passes unchanged through ALU to RZ

- Stage – 4: RY gets value from RZ

- Stage – 5: R5 gets value from RZ

Values Now:

R0: 1        R1: 3         R2: 7         R3: 6         R4: 2        R5: 2         R6: 2

And PC is incremented to next instruction :

- 80080000   in binary is         100 00000 00001 0000000000000000000

i.e. ADD R0, R1            or R1  [R0] + [R1]

- Stage – 1: Fetch to IR

Similar to above

- Stage – 2: RA, RB get their values from RF

- Stage – 3: ALU performs addition

- Stage – 4: RY gets value from RZ

- Stage -5: R1 gets value from RY and PC++

i.e. R1  1 + 3 (=4)

Values Now:

R0: 1        R1: 4         R2: 7         R3: 6         R4: 2        R5: 2        R6: 2

- a3200000   in binary is         101 00011 00010 0000000000000000000

i.e. SUB R3, R4            or R4  [R3] - [R4]

- Stage – 1: Fetch to IR

- Stage – 2: RA, RB get their values from RF

- Stage – 3: ALU performs Subtraction

- Stage – 4: RY gets value from RZ

- Stage -5: R4 gets value from RY and PC++

i.e. R4  6 - 2 (=4)

Values Now:

R0: 1        R1: 4         R2: 7         R3: 6         R4: 4        R5: 2        R6: 2

- c3280000   in binary is         110 00011 00101 0000000000000000000

i.e. MUL R3, R5            or R5  [R3] * [R5]

- Stage – 1: Fetch to IR

- Stage – 2: RA, RB get their values from RF

- Stage – 3: ALU performs Multiplication

- Stage – 4: RY gets value from RZ

- Stage -5: R5 gets value from RY and PC++

i.e. R5  6 * 2 (=12)

Values Now:

R0: 1        R1: 4         R2: 7         R3: 6         R4: 4        R5: 12        R6: 2

- e5080000   in binary is         111 00101 00001 0000000000000000000

i.e. DIVIDE R5, R1            or R1  [R5] / [R1]

- Stage – 1: Fetch to IR

- Stage – 2: RA, RB get their values from RF

- Stage – 3: ALU performs Division

- Stage – 4: RY gets value from RZ

- Stage -5: R1 gets value from RY and PC++

i.e. R5  12 / 4 (=3)

Values Now:

R0: 1        R1: 3         R2: 7         R3: 6         R4: 4        R5: 12        R6: 2

- 0008000a   in binary is         000 00000 00001 0000000000000001010

i.e. LOAD R0, R1, #10            or R1  [ [R0] +  10 ]

- Stage – 1: Fetch to IR

- Stage – 2: RA get their values from RF, RB gets IMMEDIATE value

- Stage – 3: ALU performs Addition, RY gets the address to be read from

- Stage – 4: RY gets value from the ROM/RAM at address in RY

- Stage -5: R0 gets value from RY and PC++

i.e. R0  [ [1 + 10] ] which is 0000b location on ROM and the value there  is

000000011 , therefore R0 <- 00000011 or 17

Values Now          R0: 1        R1: 17         R2: 7         R3: 6         R4: 4        R5: 12        R6: 2

- 202c0000   in binary is         001 00000 00010 1000000000000000000

i.e. STORE R0, R5, #0     or   [R0] +  0  [R5]  i.e. RAM gets written at this location by [R5]

- Stage – 1: Fetch to IR

- Stage – 2: RA get their values from RF, RB gets IMMEDIATE value

- Stage – 3: ALU performs Addition, RY gets the address to be write  onto, RM gets value of R5

- Stage – 4: Value of RM is written on the address in RY

- Stage -5: PC++

- 6000000f   in binary is         011 00000 00000 00000000000000001111

i.e. JUMP IMMEDIATE     or   PC  location 0000f on ROM

- Stage – 1: Fetch to IR

- Stage – 2: -

- Stage – 3: MUX-PCSet is Enabled and IMMEDIATE is Selected

- Stage – 4: PC gets value of IMMEDIATE

- Stage -5: PC++

Now the further instructions are executed from this point onwards.